Mid Term Project Report

Introduction to Computer Vision 2020

Daniel Ivkovich: 316421262

15/05/2020

Abstract

Convolutional Networks are based on the correlation operation (convolution without 180 degrees rotation of the filter kernel).

In this work I have wrote and used the basic convolution operation, including Stride, Padding and normalized cross correlation.

The objective is to create the forward path of a convolutional layer (activation map), by adding to the convolution (correlation) Stride, Padding and Normalization (Normalized Cross Correlation to enable template matching), followed by an activation non linearity. Then to apply multiple kernels to a single input image / activation map.

The output image should have the correct size / dimensions, based in the input size and hyper parameters and the operation will work on all types of images: grey level / RGB / Float / UINT8.

Introduction

Motivation: The Convolution/Correlation function is a very important feature in Computer Vision. It constitutes the basis for a Convolutional Neural Network which is used to analyze, categorize and create images.

It is very important to implement the Convolution in a precise manner because inside a neural network we can't always keep track on what's going on and it is important to forward correct and precise information forward.

We can use the function in order to extract vital information about an image for future use, this information is called features and we can do this while ignoring the things we don't want in the image.

These Features will then move forward in the Neural network and will be used to do what the network does.

We can also use the Convolution/Correlation function in order to find specific patches of information in a picture, we can use a small sub-image as the Filter to which we will use on the original image using a special normalization called Normalized cross validation – which is a form of similarity between the filter and the original image in each iteration.

The Convolution will result in a new image with higher frequencies in the most matching areas. This output can be threshholded and we will be left with white dots in the areas that Match the pattern almost correctly or prefectly.

scipy has a function called conv2d() which calculates the Convolution and a function called correlate2d() which calculates the Correlation.

These functions do not work on all types of images and need improvement working with  RGB Float / RGB UINT8.

Moreover, they do not have the option to use Normalized Cross Correlation which I have Implemented in this project.

Hyperparameters include: stride, padding, Norm, ACTV.

stride and padding influence the output size by the following template:

H1 = (H – F + 2*p)//stride + 1.

W1 = (W – F + 2*p)//stride + 1.

Proposed Solution

Stage One:

Define shapes and Output Dimention according. ( W1, H1, N)

Check the input shapes and deal with them accordingly.

If they are GRAYSCALE then add a third axis to them.

Stage Two:

if the boolean value CORR is False: flip all kernels by 180 degrees.

Stage Three:

Calculate N times:

Convolve Each channel of the Image with the corresponding channel of the filter, taking into account the new hyper-parameter Norm.

If Norm is activated the function will Normilize Cross corelate every patch in each iteration.

If the ACTV is present in the input as another hyper-parameter the function shall then change the output according to the activation function presented.

In Each Channel Convolution between the Image and the Filter the output will be of shape (W1, H1, 1). After performing them all and adding to a numpy array the result Output will be (W1, H1, N).

Note: if we wish to find a patch in an image we will use

norm= normalized_cross_correlation as an hyperparameter and pass it through a threshhold.

Results

Detect the parrot Eye using normalized cross validation:

Detect the Einstein's Eye using normalized cross validation:

10 Filters on Lena:

10 Filters on ImageNet grapes Images:

An Example of Bad Hyper-parameters for the Mnist Digit:

Summary And Conclusion

In conclusion to this project, the problem was to create an improved and argument versatile Convolution/Correlation function and to create a basic Convolutional Layer resulting with better feature extraction.

In addition I have imlemented a RelU activation function as well as a normalized cross validation option to the myconv function which allows the possibility to search for objects in the image and detecting their location in the image after thresholding the normalized image.

I have also proved that my convolution works precisely in the following check:

I have convolved an original image with 3x3x3, 5x5x3, 7x7x3 identity filters with the corresponding hyper parameter P = 1, 2, 3 for each filter size to get the exact same image dimentions as the input of the original image.

I have gotten a difference of zero between each convolved image and the original image, showing that the convolution is precise.

Using the convolution I have cascaded 2 layers of the Afeka Logo and thus combined 2 types of filters in a row resulting in more complex feature selection.

I have also Found Einstein's eye using the normalized cross validation as shown in the results section by convolving the image with an 15x15 eye patch of the original image.

All of the things I have implemented in this project will surely be used in the near future to implement more complex Convolutional Neural Networks for better feature extraction and better results.

Thanks for reading,

Daniel Ivkovich.

CODE IN READABLE FORMAT

ignore this if already read.

# Imports¶

In [0]:

import numpy as np

import math

from matplotlib import pyplot as plt

import cv2

from urllib.request import urlopen

# Basic Image Converts and Imshow

In [0]:

class Image(object):

@staticmethod

def show(img, title='image'):

plt.imshow(img, cmap='gray')

plt.show()

@staticmethod

def show_all(image_list, title_list):

plt.figure(figsize=[20, 20])

assert len(image_list) == len(title_list), "Houston we've got a problem"

N = len(image_list)

for index, (img, title) in enumerate(zip(image_list, title_list)):

plt.subplot(1, N, index+1)

if len(img.shape) != 3:

plt.imshow(img, cmap='gray')

else:

plt.imshow(img)

plt.title(title)

plt.show()

# Defining Helper Functions for myconv and Normalized Cross Validation

In [0]:

def show_after_conv(convolved_, labels):

convolved_gray = [norm_abs(convolved_[:, :, i]) for i in range(convolved_.shape[2])]

Image.show_all(convolved_gray, labels)

def equals(img1, img2):

img1 = normUINT8(img1)

img2 = normUINT8(img2)

boolx = np.linalg.norm((img1 - img2))

if boolx == 0:

print('The Images are equal')

else:

print('The Images are NOT equal')

def threshhold(img, thresh):

result = np.copy(img)

for i in range(img.shape[0]):

for j in range(img.shape[1]):

if img[i][j] < thresh:

result[i][j] = 0

return result

def norm(img):

'''

converts to Float and between [0, 1]

'''

img = img.astype(float)

img -= img.min()

max_val = img.max()

if max_val > 0:

img /= img.max()

return img

def normUINT8(img):

img = img.astype(float)

img -= img.min()

img /= img.max()

img *= 255

return img.astype('uint8')

def read_image_from_link(link, size):

data = urlopen(link).read()

data = np.frombuffer(data, np.uint8)

rv = cv2.imdecode(data, cv2.IMREAD_COLOR)

rv = cv2.cvtColor(rv, cv2.COLOR_BGR2RGB)

rv = cv2.resize(rv, (size[0],size[1]))

return rv

def relu(img):

for i in range(img.shape[0]):

for j in range(img.shape[1]):

if img[i][j] < 0:

img[i][j] = 0

def norm_abs(img):

return (img / np.max(np.abs(img)))

def normalized_cross_validation(filt, patch):

F = filt.shape[0]

C = filt.shape[2]

assert F == patch.shape[0], 'Filter is: ' + str(filt.shape) +' != ' + 'sub Image is: ' + str(patch.shape)

assert F == patch.shape[1], 'Filter is: ' + str(filt.shape) +' != ' + 'sub Image is: ' + str(patch.shape)

patch_mean = np.mean(patch)

filter_mean = np.mean(filt)

patch_mean_substracted        = patch-patch_mean

new_filter_mean_substracted   = filt-filter_mean

squared_sum_patch = 0

squared_sum_filter = 0

for i in range(F):

for j in range(F):

for c in range(C):

squared_sum_patch += (patch_mean_substracted[i][j][c]**2)

squared_sum_filter += (new_filter_mean_substracted[i][j][c]**2)

total_div = math.sqrt(squared_sum_patch * squared_sum_filter)

normalized_patch = patch_mean_substracted/total_div

return normalized_patch

def filter_iteration(filt, patch):

"""

Filter = FxFxC

patch = FxFxC

returns the sum of the Iteration.

"""

F = filt.shape[0]

C = filt.shape[2]

assert F == patch.shape[0], 'Filter is: ' + str(filt.shape) +' != ' + 'sub Image is: ' + str(patch.shape)

assert F == patch.shape[1], 'Filter is: ' + str(filt.shape) +' != ' + 'sub Image is: ' + str(patch.shape)

result = 0.0

for i in range(F):

for j in range(F):

result += (filt[i, j, :] * patch[i, j, :]).sum()

return result

# Defining myconv Function

In [0]:

def myconv(img, ker, stride=1, p=0, N=1, Norm=None, CORR=False, ACTV=None):

"""

ker =  FxFxC

img   = WxHxC

output  = W1xH1x1

RETURNS outputs = W1xH1xN

"""

ker = ker.astype(float)

if len(img.shape) == 2:

img = img[:, :, np.newaxis]

if len(ker.shape) == 2:

ker = ker[:, :, np.newaxis]

F = ker.shape[0] # kernel dimentions are FxF

C = ker.shape[2] # Filter channels

W = img.shape[0]

H = img.shape[1]

H1 = (H - F + 2*p)//stride + 1

W1 = (W - F + 2*p)//stride + 1

outputs = np.zeros((W1, H1, N))

# pad image with mirror mode.

PADDED_IMG = np.pad(norm(img), pad_width=((p, p), (p, p), (0,0)), mode='constant', constant_values=0)

# Flip Kernels if Convolution mode is enabled.

if not CORR:

for c in range(C):

kernel = ker[:,:, c]

kernel = np.flip(np.flip(kernel, axis=0),axis=1)

# Apply N filters on the Image.

for i in range(N):

output_shape = (W1, H1)

output = np.zeros(output_shape) # (W1,H1,C)

width_end = W - (F//2 + 1) + 2*p - (F//2 - 1)

height_end = H - (F//2 + 1) + 2*p - (F//2 - 1)

# Convolution Operation Loops

for y in range(0, width_end, stride):

for x in range(0, height_end, stride):

start_y = y

end_y = start_y + F

start_x = x

end_x = start_x + F

sub_image = PADDED_IMG[start_y:end_y, start_x:end_x, :] # (F, F, C)

if Norm is not None:

sub_image = normalized_cross_validation(ker, sub_image)

result_pixel = filter_iteration(ker, sub_image)      # (F, F, C) , # (F, F, C)

output[y//stride][x//stride] = result_pixel  # shape (1) being put W1,H1 times. total: (W1, H1, 1)

if ACTV is not None:

ACTV(output) # Now a single output is W1,H1,1

outputs[:, :, i] = output

return outputs

# Get Desired Images from the Web

In [0]:

lena_url = 'https://www.researchgate.net/profile/Adelrahman_Idries/publication/232815017/figure/fig3/AS:300463506378754@1448647490218/Original-Lena-image.png'

afeka_url = 'https://www.afeka.ac.il/images/default.jpg'

mnist_url = 'https://datawrangling.s3.amazonaws.com/sample_digit.png'

imageNet_url = 'http://www.image-net.org/nodes/4/12492106/d2/d2b59602c09e7a5931b8cc2defe738211e015e73.thumb'

waldo_url = 'https://static.boredpanda.com/blog/wp-content/uploads/2020/03/where-is-waldo-coronavirus-edition-book-fb26.png'

parrot_url = 'https://spca.bc.ca/wp-content/uploads/bird-parrot-scarlet-macaw-on-branch-in-wild.jpg'

albert_url = 'https://www.biography.com/.image/t_share/MTE5NDg0MDU0OTU2OTAxOTAz/albert-einstein-9285408-1-402.jpg'

grapes_url = 'http://www.image-net.org/nodes/4/07745940/25/256dac393e9ac7e52d236d83f7f90fc52711756d.thumb'

grapes2_url = 'http://www.image-net.org/nodes/8/07744811/9a/9a6e15cc9f8903a9d6d663a6f156d9a17fb8a6b0.thumb'

grapes3_url = 'http://www.image-net.org/nodes/8/07744811/b9/b90c1d496a9957103f1234480f3ed0bbf30c5a59.thumb'

grapes4_url = 'http://www.image-net.org/nodes/8/07744811/ba/bae4759e81d998b5145d6979d6d47e7e75b48181.thumb'

In [170]:

lena_img = read_image_from_link(link=lena_url, size=(200, 150))

afeka_img = read_image_from_link(link=afeka_url, size=(400, 200))

mnist_img = read_image_from_link(link=mnist_url, size=(28, 28))

imageNet_img = read_image_from_link(link=imageNet_url, size=(200, 250))

waldo_img = read_image_from_link(link=waldo_url, size=(150, 150))

albert_img = read_image_from_link(link=albert_url, size=(100, 100))

parrot_img = read_image_from_link(link=parrot_url, size=(100,100))

grapes_img = read_image_from_link(link=grapes_url, size=(250,300))

grapes2_img = read_image_from_link(link=grapes2_url, size=(220,190))

grapes3_img = read_image_from_link(link=grapes3_url, size=(250,200))

grapes4_img = read_image_from_link(link=grapes4_url, size=(200,240))

imgs = [lena_img, afeka_img, mnist_img, imageNet_img, waldo_img, albert_img, parrot_img, grapes_img, grapes2_img, grapes3_img, grapes4_img]

Image.show_all(imgs, ['' for i in range(len(imgs))])

# Define Convolutional Layer using myconv function

In [0]:

class Convolution_layer():

def __init__(self, data, ker, s, p, n, norm=None, actv=None):

'''

FILTER  =   FxFxC

INPUT   =   HxWxC

NORM    =   None or normalized cross validation

ACTV    =   None or RelU

'''

self.data     = data

self.ker      = ker

self.s        = s

self.p        = p

self.n        = n

self.norm     = norm

self.actv     = actv

def forward(self):

'''

RETURNS output of shape: W1xH1xN

'''

return myconv(img=self.data, ker=self.ker, stride=self.s, p=self.p, N=self.n, Norm=self.norm, ACTV=self.actv)

# Kernels and Filters for later

In [0]:

"""  KERNELS AND FILTERS"""

# kernels FxF

nothing3D = np.array([[0, 0, 0],[0, 1, 0], [0, 0, 0]])

nothing5D = np.zeros((5,5))

nothing5D[2][2] = 1

nothing7D = np.zeros((7,7))

nothing7D[3][3] = 1

sobel_x = np.array([[1, 2, 1],[0, 0, 0],[-1, -2, -1]])/5

sobel_y = np.array([[1, 0, -1],[2, 0, -2],[1, 0, -1]])/5

blur = np.array([[1, 1, 1],[1, 1, 1],[1, 1, 1]])/9

blur_filter = np.ones((3, 3, 3))/27

sharpen = np.array([[-1/9,-1/9,-1/9],[-1/9,3,-1/9],[-1/9,-1/9,-1/9]])

edge_detection = np.array([[-1,-1, -1],[-1, 8, -1],[-1, -1, -1]])

up_down = np.array([[0, 1, 0],[0, 0, 0],[0, 1, 0]])

degree45_line_detector = np.array([[0, -2, -1],

[2, 0, -2],

[1, 2, 0]])/5

degree_minus45_line_detector = np.array([[1, 2, 0],

[2, 0, -2],

[0, -2, 1]])/(-5)

red_kernel = np.zeros((3,3))

blue_kernel = np.zeros((3,3))

# Filters FxFx3 and FxFx5 and FxFx7

blur_5D = np.ones((3, 3, 5))

identity_5D = np.zeros((3, 3, 5))

identity_5D[1][1][:] = np.ones((5))

degree_minus_45_filter = np.repeat(degree45_line_detector[:, :, np.newaxis], 3, axis=2)

degree45_filter = np.repeat(degree_minus45_line_detector[:, :, np.newaxis], 3, axis=2)

red_filter = np.repeat(red_kernel[:, :, np.newaxis], 3, axis=2)

red_filter[1][1][0] = 1

blue_filter = np.repeat(blue_kernel[:, :, np.newaxis], 3, axis=2)

blue_filter[1][1][2] = 1

identity = np.repeat(nothing3D[:, :, np.newaxis], 3, axis=2)

up_down_filter = np.repeat(up_down[:, :, np.newaxis], 3, axis=2)

sharpen_filter = np.repeat(sharpen[:, :, np.newaxis], 3, axis=2)

edge_detection_filter = np.repeat(edge_detection[:, :, np.newaxis], 3, axis=2)

blur_filter = np.repeat(blur[:, :, np.newaxis], 3, axis=2)

sobel_y_filter = np.repeat(sobel_y[:, :, np.newaxis], 3, axis=2)

sobel_x_filter = np.repeat(sobel_x[:, :, np.newaxis], 3, axis=2)

blacken_filter = np.zeros((3, 3, 3))

filters1 = [sharpen_filter, edge_detection_filter, sobel_x_filter, sobel_y_filter, red_filter]

filters2 = [degree45_filter, degree_minus_45_filter, up_down_filter, blur_filter, blue_filter]

labels1 = ["sharpen filter", "edge detection filter", "sobel_x filter", "sobel_y filter", "red filter"]

labels2 = ["degree45_filter", "degree_minus_45_filter", "up_down_blur", "box blur", "blue filter"]

def forward_10_filters_and_show(img, s, p, n, actv):

global filters1

global filters2

global labels1

global labels2

image_set1 = []

image_set2 = []

for filt in FILTERS1:

conv_layer = Convolution_layer(data=img, ker=filt, s=s, p=p, n=n, actv=actv)

image_set1.append(conv_layer.forward()[:, :, 0])

for filt in FILTERS2:

conv_layer = Convolution_layer(data=img, ker=filt, s=s, p=p, n=n, actv=actv)

image_set2.append(conv_layer.forward()[:, :, 0])

Image.show_all(image_set1, LABELS1)

Image.show_all(image_set2, LABELS2)

# Cascading 2 Convolutional Layers and Demonstrating the results.

Note: the first layer produces W1xH1x3 Therefore, the second filter should be FxFx3 and then goes through the 2nd layer and finaly outputs W1xH1x1

In [173]:

def cascade(img):

conv_layer1 = Convolution_layer(data=img, ker=blur, s=1, p=1, n=3, norm=None, actv=None)

x = conv_layer1.forward()

rv10 = x[:,:,0]

conv_layer2 = Convolution_layer(data=x, ker=sobel_x_filter, s=1, p=1, n=1, norm=None, actv=None)

x = conv_layer2.forward()

rv2 = x[:,:,0]

Image.show_all([rv10, rv2], ['first layer','second layer'])

cascade(afeka_img)

# Convolving gray parrot Image with 3x3x3 5x5x3 7x7x3 identity filters only adjusting the padding so the output will be of the same dimention.

# All the Images are Equal which proves that the Convolution is working.

In [174]:

gray_parrot = cv2.cvtColor(parrot_img, cv2.COLOR_RGB2GRAY)

labels = ['Original', 'nothing3D', 'nothing5D', 'nothing7D']

convolved1 = myconv(img=gray_parrot, ker=nothing3D, stride=1, p=1)[:,:,0]

convolved2 = myconv(img=gray_parrot, ker=nothing5D, stride=1, p=2)[:,:,0]

convolved3 = myconv(img=gray_parrot, ker=nothing7D, stride=1, p=3)[:,:,0]

convolved = [convolved1, convolved2, convolved3]

for conv in convolved:

equals(gray_parrot, conv)

Image.show_all([gray_parrot, convolved1, convolved2, convolved3], labels)

The Images are equal

The Images are equal

The Images are equal

# Using Alberts Eye Pattern as Filter with Normalized Cross Validation and Threshholding Image to detect Matching patterns

# Note: the padding is 7 because the filter is 9x9 and I wanted the Result to be 'same' dimentions.

In [175]:

albert_gray = cv2.cvtColor(albert_img, cv2.COLOR_RGB2GRAY)

albert_eye_filter = albert_gray[50:65, 30:45]

convolved = myconv(img=albert_gray, ker=albert_eye_filter, stride=1, p=7, N=1, Norm=normalized_cross_validation, ACTV=relu)

convolved_gray = convolved[:, :, 0]

result = convolved_gray

thresh = threshhold(result, 0.8)

Image.show_all([albert_gray, albert_eye_filter, result, thresh], ['Original', 'Eye Pattern', 'Result', 'Result Threshholded'])

# Using Parrot Eye Pattern as Filter and Threshholding Image to detect Matching patterns

# Note: padding is 5 because the filter is 5x5 and I wanted the result to be of 'same' dimentions.

In [176]:

parrot_eye_filter = parrot_img[25:40, 30:45]

convolved = myconv(img=parrot_img, ker=parrot_eye_filter, stride=1, p=5, N=1, Norm=normalized_cross_validation, ACTV=relu)

convolved_gray = convolved[:, :, 0]

result = convolved_gray

thresh = threshhold(result, 0.8)

Image.show_all([parrot_img, parrot_eye_filter, result, thresh], ['Original', 'Eye Pattern', 'Result', 'Result Threshholded'])

# An Example of Bad Hyper-parameters

We can't even see that the digit is '4'

In [177]:

convolved = myconv(img=mnist_img, ker=sharpen_filter, stride=8, p=50, N=1, Norm=None, ACTV=None)

convolved_gray = convolved[:, :, 0]

Image.show(convolved_gray)

# Operating N=10 Filters on the same Image with different hyper-parameters using the Convolutional Layer I made.

In [178]:

forward_10_filters_and_show(img=lena_img, s=1, p=1, n=1, actv=None)

In [179]:

forward_10_filters_and_show(img=waldo_img, s=1, p=5, n=1, actv=None)

In [180]:

forward_10_filters_and_show(img=albert_img, s=2, p=3, n=1, actv=relu)

In [181]:

forward_10_filters_and_show(img=parrot_img, s=1, p=5, n=1, actv=relu)

In [182]:

forward_10_filters_and_show(img=grapes_img, s=1, p=2, n=1, actv=None)

In [183]:

forward_10_filters_and_show(img=grapes2_img, s=1, p=2, n=1, actv=None)

In [184]:

forward_10_filters_and_show(img=grapes3_img, s=1, p=2, n=1, actv=None)